Problem Statement + Data Understanding

WHY are we building this?

General Aviation mein — flight schools, small charter operators, or even at any airport — aircraft maintenance aaj bhi reactive hai.

- Matlab jab tak kuch toot nahi jaata, tab tak koi action nahi hota.

- Ya phir calendar-based hai — "500 hours ho gaye, chalao maintenance.

Dono approaches mein problem yeh hai:

- Reactive → aircraft already compromised ho chuka hota hai jab pata chalta hai. Safety risk + emergency grounding = revenue loss.

- Calendar-based → sometimes part bilkul theek hota hai fir bhi replace kar do, ya kabhi kabhi part 480 hours pe hi deteriorate ho jaata hai but next check 500 pe hai.

Jo data already exist karta hai — har flight ke dauran Cessna 172 ka avionics system har second 23+ sensors ki readings record karta hai. Oil temperature, RPM, airspeed, altitude, engine parameters — sab kuch. Yeh data NGAFID (National General Aviation Flight Information Database) mein jata hai.

AeroGuard ka kaam — is per-second sensor data ko lekar predict karna ki:

- Kya yeh aircraft agli 2 din mein maintenance maangega? (RUL prediction)

- Agar haan, toh kaunsa part fail hone wala hai? (Fault Classification)

- Kaunse flight segment mein anomaly thi? (Anomaly Detection)

- Kyun yeh prediction aayi — sensor level explanation with plain language (XAI)

DATASET –

Dataset mein 2 main files hain:

- flight_header.csv        ← ek row = ek flight ki summary

- all_flights/one_parq/    ← actual per-second sensor readings (Dask Parquet)

DATA DICTIONARY -

FLIGHT HEADER — Har Column ka Matlab

| Column | Matlab Project Mein Use | Description |
| --- | --- | --- |
| Master Index | Primary join key | Har flight ka unique ID — dono files ko join karne ki key |
| tail_num | Aircraft-wise grouping, leakage prevention | Aircraft registration number (jaise car ka number plate) |
| start_time | Temporal ordering, season feature | Flight shuru hone ka timestamp |
| end_time | Flight khatam hone ka timestamp | Flight duration calculate karna |
| duration | Filter karna — 30 min se kam flights remove | Total flight time (seconds) |
| before_maintenance | Binary RUL classification | Target variable #1 — 1 matlab yeh flight maintenance se PEHLE thi |
| maintenance_type | Multi-class fault classification | Target variable #2 — 36 types mein se kaunsa issue (cluster ID) |
| days_until_maintenance | RUL regression (optional) | Flight ke baad kitne din mein maintenance hua |
| aircraft_type | Confirmation/filter | Cessna 172S — sab same hain is dataset mein |
| maintenance_date | Temporal feature engineering | Actual jis din maintenance hua |

Critical concept — Data Leakage: tail_num se hum aircraft ko track karte hain. Train/test split mein same aircraft ke flights dono sets mein nahi aane chahiye — warna model "memorize" kar lega specific aircraft ka behavior, generalize nahi karega.

FLIGHT DATA — 23 Sensor Parameters ka Matlab

Yeh Cessna 172 ka avionics system hai — har second ek row. Ek 1-hour flight = 3600 rows sirf ek aircraft ke liye.

Engine Parameters — sabse critical for maintenance prediction:

| Sensor | Full Name | Description | Normal Range | Maintenance Signal |
| --- | --- | --- | --- | --- |
| E1 RPM | Engine 1 RPM | Propeller kitni tezi se ghoom raha hai | 600–2700 RPM | Irregular fluctuation = ignition ya fuel issue |
| E1 CHT1-4 | Cylinder Head Temp 1-4 | Engine ke 4 cylinders ka temperature | 200–400°F | Sustained high temp = cooling ya valve issue |
| E1 EGT1-4 | Exhaust Gas Temp 1-4 | Exhaust gases ka temperature | 1200–1600°F | Imbalance across cylinders = fuel mixture problem |
| E1 OilT | Engine Oil Temperature | Engine oil kitna garam hai | 100–245°F | High oil temp = lubrication failure warning |
| E1 OilP | Engine Oil Pressure | Oil system ka pressure | 25–100 PSI | Low pressure = critical failure imminent |
| E1 FFlow | Fuel Flow | Kitna fuel engine consume kar raha hai | 4–12 GPH | Abnormal flow = injector/carburetor issue |
| E1 MAP | Manifold Air Pressure | Engine intake mein air pressure | 15–30 inHg | Indicates engine load and throttle position |

Flight Parameters — aircraft ki overall state:

| Sensor | Full Name | Description | Normal Use | Maintenance Relevance |
| --- | --- | --- | --- | --- |
| IAS | Indicated Airspeed | Aircraft ki actual speed jo pilot dekhta hai | Flight envelope monitoring | Normal flight envelope check |
| AltMSL | Altitude Mean Sea Level | Samudra se kitni upar hai aircraft | Standard altitude reference | Flight phase identification |
| AltAGL | Altitude Above Ground Level | Zameen se actual height | Ground proximity monitoring | Takeoff/landing detection |
| VSpd | Vertical Speed | Kitni tezi se upar ya neeche ja rahi hai | Climb/descent management | Climb/descent rate anomaly detection |
| Pitch | Pitch Angle | Nose upar ya neeche ka angle | Attitude control | Unusual attitude detection |
| Roll | Roll Angle | Wing left ya right jhukne ka angle | Bank angle control | Excessive bank angle monitoring |
| HDG | Heading | Compass direction jis taraf ja rahi hai aircraft | Navigation reference | Unusual heading deviation |
| LatAc | Lateral Acceleration | Side-to-side force jo aircraft par lagti hai | Maneuver monitoring | Turbulence, rough handling detection |
| NormAc | Normal Acceleration | Vertical G-force jo airframe par padti hai | Structural load monitoring | G-force on airframe stress detection |

Navigation/GPS:

| Sensor | Full Name | Matlab Use |
| --- | --- | --- |
| Latitude/Longitude | Geographic Position Coordinates | Aircraft ki exact location track karna — GPS-based position monitoring |
| GndSpd | Ground Speed | Actual speed over ground — wind effect include hoti hai, airspeed se alag hoti hai |
| WndSpd/WndDr | Wind Speed & Direction | External environmental factor — flight performance aur fuel consumption pe impact |

General Project Structure for this project –

aeroguard/

│

├── .env

├── .gitignore

├── requirements.txt

├── README.md

├── docker-compose.yml

├── Dockerfile

│

├── configs/

│   └── config.yaml

│

├── src/

│   ├── __init__.py

│   ├── exception.py

│   ├── logger.py

│   │

│   ├── data/

│   │   ├── ingestion.py

│   │   ├── validation.py

│   │   └── transformation.py

│   │

│   ├── models/

│   │   ├── rul_classifier.py

│   │   ├── fault_classifier.py

│   │   └── anomaly_detector.py

│   │

│   ├── xai/

│   │   ├── shap_explainer.py

│   │   ├── attention_viz.py

│   │   └── counterfactual.py

│   │

│   ├── monitoring/

│   │   ├── mlflow_tracker.py

│   │   └── metrics.py

│   │

│   └── pipeline/

│       ├── training_pipeline.py

│       └── prediction_pipeline.py

│

├── api/

│   ├── main.py

│   ├── routes/

│   │   ├── predict.py

│   │   ├── explain.py

│   │   └── fleet.py

│   └── schemas.py

│

├── dashboard/

│   ├── app.py

│   ├── mechanic_view.py

│   └── fleet_view.py

│

├── notebooks/

│   └── 01_eda.ipynb

│

├── tests/

│   ├── test_ingestion.py

│   ├── test_models.py

│   └── test_api.py

│

├── mlruns/

├── logs/

└── artifacts/

COMPLETE DATA UNDERSTANDING –

Dataset ka Size aur Scale

| Metric | Value |
| --- | --- |
| Total flight hours | 31,177 hours |
| Total flights | 28,935 flights |
| Unplanned maintenance events | 2,111 events |
| Maintenance issue types (clusters) | 36 types |
| Sensors | 23 sensors |
| Sampling rate | Har second ek row |
| Total rows | 100 million+ rows |
| Dataset size | 4.3 GB |
| Aircraft model | Cessna 172 only |
| Source | Single flight school, multiple years |

Poore 23 Sensors — Ek Ek Ka Matlab –

⚡ Electrical System

| Sensor | Full Name | Matlab | Normal Behavior | Failure Signal |
| --- | --- | --- | --- | --- |
| volt1 | Main Electrical Bus Voltage | Main battery + alternators ka voltage | 13.8–14.5V (charging state mein) | Low voltage = alternator failure |
| volt2 | Essential/Standby Bus Voltage | Backup battery ka voltage | 13.5–14V (normal ops mein) | Sudden drop = standby battery issue |
| amp1 | Main Battery Ammeter | Positive = charging, Negative = discharging | Positive value during normal flight | Sustained negative = alternator not charging |
| amp2 | Standby Battery Ammeter | Backup battery ka same charging/discharging indicator | Near zero during normal operations | Unusual drain = electrical fault detected |

⛽ Fuel System

| Sensor | Full Name | Matlab | Normal Range | Failure Signal |
| --- | --- | --- | --- | --- |
| FQtyL | Fuel Quantity Left Tank | Left wing tank mein kitna fuel bacha hai | 0–26 gallons | Rapid drop = fuel leak suspected |
| FQtyR | Fuel Quantity Right Tank | Right wing tank mein kitna fuel bacha hai | 0–26 gallons | Asymmetric drain = fuel selector issue |
| E1 FFlow | Engine Fuel Flow Rate | Engine kitna fuel consume kar raha hai per hour | 4–12 GPH | Abnormal flow = injector ya carburetor issue |

Engine — Most Critical for Maintenance

| Sensor | Full Name | Matlab | Normal Range | Failure Signal |
| --- | --- | --- | --- | --- |
| E1 OilT | Engine Oil Temperature | Engine oil kitna garam hai | 100–245°F | Sustained high = cooling failure ya low oil level |
| E1 OilP | Engine Oil Pressure | Oil system ka pressure | 25–100 PSI | Low pressure = CRITICAL — engine seizure risk |
| E1 RPM | Engine RPM | Propeller + crankshaft ki rotation speed | 600–2700 RPM | Rough/fluctuating = ignition ya fuel delivery issue |
| E1 CHT1 | Cylinder 1 Head Temp | Cylinder 1 ke head ka temperature | 200–400°F | High = cooling issue, lean mixture |
| E1 CHT2 | Cylinder 2 Head Temp | Cylinder 2 ke head ka temperature | 200–400°F | Imbalance across cylinders = uneven combustion |
| E1 CHT3 | Cylinder 3 Head Temp | Cylinder 3 ke head ka temperature | 200–400°F | Imbalance across cylinders = uneven combustion |
| E1 CHT4 | Cylinder 4 Head Temp | Cylinder 4 ke head ka temperature | 200–400°F | Imbalance across cylinders = uneven combustion |
| E1 EGT1 | Exhaust Gas Temp Cylinder 1 | Cylinder 1 ka exhaust temperature | 1200–1600°F | Divergence = fuel mixture ya valve issue |
| E1 EGT2 | Exhaust Gas Temp Cylinder 2 | Cylinder 2 ka exhaust temperature | 1200–1600°F | Divergence = fuel mixture ya valve issue |
| E1 EGT3 | Exhaust Gas Temp Cylinder 3 | Cylinder 3 ka exhaust temperature | 1200–1600°F | Divergence = fuel mixture ya valve issue |
| E1 EGT4 | Exhaust Gas Temp Cylinder 4 | Cylinder 4 ka exhaust temperature | 1200–1600°F | Divergence = fuel mixture ya valve issue |

NOTE: Latitude, Longitude, tail number, aur date — yeh sab privacy ke liye REMOVE kar diye gaye hain public dataset se. Matlab hum aircraft ko individually track nahi kar sakte directly — lekin master_index se flight-to-maintenance link kar sakte hain.

🏷️ Flight Header — Har Column

| Column | Type | Matlab |
| --- | --- | --- |
| Master Index | Integer (Primary Key) | Flight ka unique ID — sensor data se join key |
| before_maintenance | Binary (0/1) | Main target — 1 = flight maintenance se pehle thi |
| maintenance_type | Integer (0–35) | Multi-class target — kaunsa maintenance issue |
| days_until_maintenance | Float | Maintenance hone mein kitne din bache the |
| duration | Integer (seconds) | Flight kitni der ki thi |
| start_time | Timestamp | Flight start — season aur time-based features ke liye |
| end_time | Timestamp | Flight end — duration calculation aur temporal features ke liye |

🏷️ 36 Maintenance Types — Complete List (Paper Appendix Se)

Yeh important hai kyunki class imbalance yahan sabse bada challenge hai:

| Class | Name | Total Flights | Subset Flights |
| --- | --- | --- | --- |
| Majority | Flights after maintenance | 10,291 | 5,844 |
| Top 1 | Intake gasket leak/damage | 4,244 | 2,098 |
| Top 2 | Rocker cover leak/loose/damage | 2,157 | 1,024 |
| Mid | Baffle crack/damage/loose/missing | 545 | 304 |
| Mid | Intake tube/bolt/seal loose | 556 | 269 |
| Mid | Baffle plug need repair | 465 | 254 |
| Mid (x30) | 30 more classes | Mostly < 300 | Many = 0 |
| Rarest | Spark plug need repair | 15 | 0 |
| Rarest | Mixture fail/need adjust | 17 | 0 |

matlab — Paper ne benchmark ke liye sirf 19 classes use kiye jinmein >= 50 flights the.

⚠️ Data Ke Known Issues (Paper mai already Acknowledged hai)

Yeh padhna bahut zaroori tha — yeh issues directly hamare preprocessing aur modeling decisions affect karenge:

1. NaN Values (~1% of all data points) Causes: sensor initialization at start of flight, sensor failure mid-flight, recording system failure. Paper ne intentionally NaN rakhhe — replace nahi kiya, taaki future researchers apni technique use kar sakein.

2. Class Imbalance — Severe Top 2 classes (intake gasket + rocker cover) + post-maintenance = 95%+ of data. Baaki 34 classes mein bahut kam flights hain.

3. Pilot Variance — Biggest Challenge Yeh dataset ka sabse unique aur tough problem hai. Paper clearly kehta hai: "a significant portion of variance in flight data is explained by pilot action and not the condition of the airplane's parts." Matlab ek pilot aggressive maneuver kare toh sensors spike honge — yeh maintenance issue nahi hai. Model ko yeh distinguish karna padega.

4. Flight Length Variability Flights 30 minutes se lekar ghanton tak ki hain. Paper ne last 4096 seconds (roughly last ~68 minutes) truncate karke use kiya. Hum isko better handle kar sakte hain.

5. Chronic vs Acute Failure Mostly chronic failures hain (gradual wear — intake gasket leaks). Acute (sudden) failures bahut rare hain. Autoencoder-based anomaly detection isliye bahut mushkil hai is dataset pe — paper ne khud warn kiya hai.

6. Privacy — No Tail Number, No Date, No GPS Hum individual aircraft track nahi kar sakte. Lekin master_index se flight-to-maintenance relationship milta hai.

| Model | Binary Accuracy | Multiclass Accuracy |
| --- | --- | --- |
| ConvMHSA | 76.0% | 52.8% |
| InceptionTime | 75.5% | 54.1% |
| MiniRocket | 59.8% | 50.4% |

Humara goal — in numbers ko beat karna + proper XAI layer add karna jo paper mein bilkul nahi hai.

Ques - How Can we Create this system as an ALERT system?

AeroGuard — Alert System + Anomaly Detection

Ans - Pehle Samjho — Yeh "Alert System" Kya Exactly Karega?

Socho ek flight school hai jisme 15 Cessna 172 hain. Rozana 40-50 flights hoti hain. Abhi kya hota hai:

Flight lands → Pilot parks aircraft → Nobody checks sensor data

→ Next day pilot takes off → Mid-flight kuch fail hota hai → Emergency

AeroGuard ke saath kya hoga:

Flight lands → System automatically last flight ka data process karta hai

→ 3 layers of analysis run hoti hain → Agar kuch suspicious mila

→ Mechanic ko alert jaata hai → "Aircraft C-172 #7, Oil system anomaly detected, 73% probability of maintenance needed within 2 days,Primary cause: E1 OilT consistently elevated in cruise phase"

→ Mechanic grounds aircraft BEFORE next flight

Yahi real-world impact hai.

┌─────────────────────────────────────────────────────────────┐

│                    INCOMING FLIGHT DATA                      │

│              (23 sensors × ~3600 seconds)                   │

└─────────────────────┬───────────────────────────────────────┘

│

┌────────────▼────────────┐

│     LAYER 1             │

│   ANOMALY DETECTION     │  ← "Kuch unusual hua is flight mein?"

│   (LSTM Autoencoder)    │

└────────────┬────────────┘

│  Anomaly Score + Flagged Segments

┌────────────▼────────────┐

│     LAYER 2             │

│   RUL CLASSIFIER        │  ← "Kya yeh aircraft 2 din mein fail hoga?"

│   (Conv-MHSA)           │

└────────────┬────────────┘

│  Failure Probability (0-100%)

┌────────────▼────────────┐

│     LAYER 3             │

│   FAULT CLASSIFIER      │  ← "Kaunsa part fail hone wala hai?"

│   (InceptionTime)       │

└────────────┬────────────┘

│  Fault Type + Confidence

┌────────────▼────────────┐

│     XAI ENGINE          │  ← "Model ne yeh kyun socha?"

│  SHAP + Attention +     │

│  Counterfactuals        │

└────────────┬────────────┘

│  Human-readable explanations

┌────────────▼────────────┐

│   ALERT GENERATOR       │  ← Final decision + severity

└────────────┬────────────┘

│

┌───────────┴───────────┐

▼                       ▼

MECHANIC VIEW           FLEET MANAGER VIEW

(Single aircraft         (All aircraft

deep dive)               risk dashboard)

Layer 1 — Anomaly Detection Ko Sahi Se Samjho

Yeh sabse important aur tricky part hai. Paper ne khud warn kiya hai ki simple autoencoder kaam nahi karega — kyunki pilot variance bahut zyada hai.

Toh hum smart anomaly detection karenge — 3 types:

Anomaly Type 1 — Sensor-Level Anomaly (Per-Sensor Statistical)

Kya detect karta hai: Kisi ek sensor ki reading apni expected range se bahar gayi.

Kaise kaam karta hai:

Normal cruise mein E1 OilT → 180-220°F hota hai

Agar ek flight mein E1 OilT → 260°F pe 8 minutes tak raha

→ Yeh sensor-level anomaly hai

Technique: Rolling z-score + IQR-based bounds, per flight phase (taxi/climb/cruise/descent) alag alag thresholds. Kyunki OilT ka normal range takeoff mein alag hota hai, cruise mein alag.

Implementation:

For each sensor:

1. Flight ko phases mein tod do (AltMSL + VSpd se)

2. Har phase ke liye historical normal distribution calculate karo

3. Naye flight mein agar koi reading > 3 sigma bahar gayi

4. Flag karo — kaunse second pe, kis sensor pe, kitna deviate kiya

Anomaly Type 2 — Pattern-Level Anomaly (LSTM Autoencoder)

Kya detect karta hai: Individual sensor theek lag raha hai, but overall pattern abnormal hai — "sensor interrelationships" badal gayi hain.

Kaise kaam karta hai:

Normal flight mein jab RPM badhta hai → FFlow bhi badhta hai → CHT thoda badhta hai. Yeh relationship fixed hoti hai. Agar ek deteriorating aircraft mein RPM badha but FFlow normally nahi badha → autoencoder ko high reconstruction error milega.

Training:

- Sirf post-maintenance (healthy) flights pe train karo

- Model seekhta hai "normal flight pattern" kya hota hai

- Reconstruction error kam = normal flight

- Reconstruction error zyada = anomaly

Inference:

- Naya flight dalo

- Model reconstruct karne ki koshish karta hai

- High error wale time segments = anomalous periods

- Yeh heatmap ban jaata hai → "minute 34-41 mein kuch unusual tha"

Kyon yeh pilot variance problem solve karta hai: Hum flight ko phases mein tod ke train karenge alag alag — taxi, climb, cruise, descent. Pilot climb aggressive kare ya smooth, dono mein alag model. Isliye pilot ka behavior confuse nahi karega maintenance signal ko.

Anomaly Type 3 — Cross-Flight Trend Anomaly (Aircraft Health Score)

Yeh sabse powerful aur novel idea hai — paper mein yeh nahi kiya.

Kya detect karta hai: Ek single flight mein kuch nahi dikhta, but last 10 flights ka trend dekho toh clearly deterioration dikh raha hai.

Aircraft #7 ki last 10 flights ka E1 OilT cruise average:

Flight -10: 195°F

Flight -9:  197°F

Flight -8:  199°F

Flight -7:  202°F

Flight -6:  204°F

Flight -5:  208°F

Flight -4:  211°F

Flight -3:  215°F

Flight -2:  219°F

Flight -1:  226°F  ← Koi ek flight mein nahi pakda jaata

but trend clearly upward hai

Technique: Linear regression slope on rolling window of last N flights per sensor. Agar slope > threshold → trend anomaly alert.

Yeh isliye powerful hai kyunki: Chronic failures (jo 95% cases hain is dataset mein) single flight mein nahi dikhte — yeh gradual hote hain. Cross-flight trend isko pakadta hai jab baaki dono layers miss kar deti hain.

Layer 2 — RUL Classifier

Input: Last flight ki 4096 seconds ki 23-sensor time series Output: Probability (0–100%) ki "kya yeh aircraft agli 2 din mein maintenance maangega" Model: Conv-MHSA (paper ka best model, 76% accuracy — hum isko beat karenge)

Alert threshold logic:

P(maintenance needed) >= 80%  →  🔴 CRITICAL — Ground immediately

P(maintenance needed) 60–79%  →  🟠 HIGH     — Inspect before next flight

P(maintenance needed) 40–59%  →  🟡 MEDIUM   — Monitor closely

P(maintenance needed) < 40%   →  🟢 NORMAL   — No action needed

Layer 3 — Fault Classifier

Input: Same flight data Output: Kaunse maintenance issue type ki probability highest hai (36 classes mein se) Yeh tab activate hota hai jab Layer 2 ne >= 60% probability di ho

Output example:

Top 3 Probable Fault Types:

1. Intake Gasket Leak/Damage     → 67% confidence

2. Engine Run Rough              → 18% confidence

3. Oil Cooler Need Maintenance   → 9% confidence

XAI Engine — Alert Ko Human-Readable Banana

Sirf "alert" nahi — kyun alert aaaya aur uss alert k peeche kya reason hai ye bhi batana.

3 Types of Explanations:

- SHAP — Sensor Importance

Alert ke liye contribution:

E1 OilT    ████████████  +0.34  (sabse bada contributor)

E1 CHT2    ██████        +0.19

E1 FFlow   ████          +0.12

E1 RPM     ██            -0.08  (reduce kar raha tha alert ko)

Plain language: "Oil temperature consistently above normal range

during cruise phase was the primary driver of this alert."

2. Attention Heatmap — Time Segment Importance

Flight timeline (0 → 3600 seconds):

Taxi  |░░░░| Climb |░░██████░| Cruise |░░░░░██████████░░░| Descent |░░░|

↑                      ↑

Risk spike             Sustained risk

at minute 12          from minute 28-51

Plain language: "Anomalous patterns detected primarily during

cruise phase (minutes 28-51) and briefly during climb (minute 12)."

3. Counterfactual — "Kya hota agar"

Current: ALERT TRIGGERED (83% failure probability)

Counterfactual: "Had E1 OilT remained below 225°F during cruise,

failure probability would have been 31% (below alert threshold).

Recommended action: Check oil level and cooling system."

Final Alert — Jo Mechanic Ko Jaata Hai

╔══════════════════════════════════════════════════════════════╗

║  🔴 CRITICAL MAINTENANCE ALERT                               ║

║  Aircraft: C172 Fleet #7  |  Flight ID: 28441                ║

║  Alert Generated: After landing, 14:23 IST                   ║

╠══════════════════════════════════════════════════════════════╣

║  RISK SCORE: 83%  |  SEVERITY: CRITICAL                      ║

║  ACTION REQUIRED: Ground aircraft — Do not fly               ║

╠══════════════════════════════════════════════════════════════╣

║  MOST LIKELY FAULT: Intake Gasket Leak/Damage (67%)          ║

║  SECONDARY:         Engine Run Rough (18%)                   ║

╠══════════════════════════════════════════════════════════════╣

║  WHY THIS ALERT?                                             ║

║  • Engine oil temperature elevated 18% above cruise normal   ║

║    (avg 241°F vs historical 204°F) — sustained 23 minutes    ║

║  • CHT2 diverging from CHT1/3/4 since last 3 flights         ║

║  • Cross-flight trend: OilT rising 3.2°F per flight          ║

║    over last 8 flights (trend anomaly)                       ║

╠══════════════════════════════════════════════════════════════╣

║  CRITICAL WINDOW: minutes 28–51 of flight (cruise phase)     ║

╠══════════════════════════════════════════════════════════════╣

║  COUNTERFACTUAL: If OilT < 225°F, risk would be 31%          ║

║  RECOMMENDATION: Inspect oil system, check for leaks,        ║

║                  verify cooling passages not blocked          ║

╚══════════════════════════════════════════════════════════════╝

Fleet Manager Dashboard

FLEET STATUS — 15 Aircraft — Last Updated: 14:25 IST

Aircraft    Last Flight    Risk Score    Status         Action

─────────────────────────────────────────────────────────────

C172 #7     2h ago         83% 🔴        GROUNDED        Inspect oil system

C172 #3     4h ago         61% 🟠        HOLD            Pre-flight inspect

C172 #12    1h ago         54% 🟡        MONITOR         Check before AM flight

C172 #1     3h ago         12% 🟢        AIRWORTHY       Clear

C172 #9     5h ago         8%  🟢        AIRWORTHY       Clear

...

FLEET HEALTH TREND (Last 30 days)

Aug: ████████░░  78% airworthy

Sep: ███████░░░  71% airworthy

Oct: █████░░░░░  52% airworthy  ← Degradation trend detected fleet-wide

PROJECT OVERVIEW –

Github Repository and cloning

Step 1 – Create a github repo “Aeroguard” and add a .gitignore file and readme file to it along with any license (for style)

Step 2 – Clone your repository to a destination on your local system and Open VS code with your project folder,check for python version –

python --version

(python version must not be less than 3.10)

Virtual Environment

Step 3 - Create a virtual environment using –

python -m venv aviation

Step 4 - Activate the virtual environment –

aviation\Scripts\activate

Project Structure –

Dependency Setup

Step 5 – create a “requirements.txt” file inside the root folder

# Core data libraries

numpy

pandas

# Data visualization

matplotlib

seaborn

# Machine learning

scikit-learn

# Large scale data processing

dask

pyarrow

# Notebook environment for EDA

jupyter

# Logging and environment management

loguru

python-dotenv

# Experiment tracking

mlflow

#for yaml

pyyaml

install the dependies using – “pip install -r requirements.txt”

Environment Variables setup –

Step 6 – create a “.env” file inside the root folder –

(root folder mein — GitHub pe kabhi push nahi hogi – isme mostly humare development related info, aur secret keys hoti hai isiliye kabhi bhi cloud pe push nahi honi chahiye)

# ============================================================

# AeroGuard — Environment Variables

# IMPORTANT: Yeh file .gitignore mein add ki gayi hai

# Yahan secrets, API keys aur paths store hote hain

# ============================================================

# Project ke main folders ke paths

PROJECT_ROOT=.

DATA_DIR=./data

ARTIFACTS_DIR=./artifacts

LOGS_DIR=./logs

# MLflow experiment tracking ke liye paths aur naam

MLFLOW_TRACKING_URI=./mlruns

MLFLOW_EXPERIMENT_NAME=aeroguard

# FastAPI server settings (baad mein use hoga)

API_HOST=0.0.0.0

API_PORT=8000

# Development ya production — behavior change hoga iske basis pe

ENVIRONMENT=development

Configuration file setup –

Step 7 – Create a “configs” folder inside the root folder “Aeroguard” and under it create a “config.yaml” file

# ============================================================

# AeroGuard — Central Configuration File

# Saari project settings yahan se control hoti hain

# Code mein koi bhi value hardcode nahi karni — yahan se leni hai

# ============================================================

project:

name: aeroguard

version: "1.0.0"

environment: development        # development ya production

paths:

data_dir: ./data                # raw dataset yahan hoga

artifacts_dir: ./artifacts      # trained models yahan save honge

logs_dir: ./logs                # log files yahan jaayengi

mlruns_dir: ./mlruns            # MLflow runs yahan store honge

data:

# NGAFID dataset ke specific paths

raw_flight_data: ./data/all_flights/one_parq    # Dask parquet files

flight_header: ./data/all_flights/flight_header.csv  # flight metadata

# 30 minutes se kam ki flights ignore karenge — too short for analysis

min_flight_duration: 1800       # seconds mein

# Paper ke benchmark ke hisaab se last 4096 seconds use karenge

truncate_timesteps: 4096

model:

# RUL = Remaining Useful Life

# Agar flight maintenance se 2 din pehle hai → label = 1 (needs maintenance)

rul_threshold: 2                # days

# Alert severity thresholds — probability ke basis pe

alert_critical: 0.80            # 80%+ → CRITICAL — turant ground karo

alert_high: 0.60                # 60-79% → HIGH — agli flight se pehle inspect

alert_medium: 0.40              # 40-59% → MEDIUM — monitor karo

logging:

level: DEBUG                    # DEBUG = sab kuch log hoga (development)

# INFO = sirf important events (production)

rotation: "10 MB"               # file 10MB hone pe nayi file banega

retention: "30 days"            # 30 din purani files automatically delete

Step 8 – Create a “src” folder inside the root folder

This would be our source folder where most of the codes related to our pipeline would reside.

Why src Layout?

Large production codebases often follow the src layout pattern.

Benefits:

- Clear separation of source code and project root

- Avoids accidental imports from local scripts

- Improves packaging and deployment reliability

- Industry standard for scalable Python systems

In this structure, all application logic resides inside the src/ directory while configuration, documentation, and environment files remain in the project root.

Step 9 – create a “__init__.py” file inside the “src” folder –

Ye file kyun important hai?

Ye Python ko batata hai:

ye folder ek Python package hai

Isse future mein hum aise import kar sakte hain:

from src.data.ingestion import load_data

Agar __init__.py nahi hota hai toh Python kabhi-kabhi folder ko package treat nahi karta (Not always but in general)

What is __init__.py?

__init__.py converts a directory into a Python package.

This enables:

- modular imports

- package level initialization

- structured project architecture

Without it, Python may not recognize the folder as an importable module.

Project Logging System –

Dono jagah Logs capture hone chahiye

✅ Terminal mein real-time

✅ logs/ folder mein file mein permanently

Step 10 – create a “logger.py” file under “src” folder

# ============================================================

# AeroGuard — Central Logger

# Yeh file poore project mein logging handle karti hai

# Do jagah simultaneously log hoga:

#   1. Terminal (color ke saath — development mein helpful)

#   2. logs/ folder mein daily rotating files

# Usage: from src.logger import logger

#        logger.info("Kuch hua")

#        logger.error("Kuch galat hua")

# ============================================================

import os

import sys

from loguru import logger

from dotenv import load_dotenv

import yaml

# .env file se environment variables load karo

load_dotenv()

# .env se logs directory path lo, agar nahi mili toh default "./logs"

LOGS_DIR = os.getenv("LOGS_DIR", "./logs")

# logs directory banao agar exist nahi karti

# exist_ok=True matlab agar pehle se hai toh error mat do

os.makedirs(LOGS_DIR, exist_ok=True)

# config.yaml se logging level padho

# iska fayda — sirf config.yaml change karo, code nahi

with open("configs/config.yaml", "r") as f:

config = yaml.safe_load(f)

LOG_LEVEL = config["logging"]["level"]  # "DEBUG" ya "INFO"

# Loguru ka default handler remove karo

# Warna duplicate logs aayenge

logger.remove()

# ---- Handler 1: Terminal (Console) ----

# Yeh developer ko real-time colored output deta hai

logger.add(

sys.stdout,

level=LOG_LEVEL,

# Format explanation:

# {time} = timestamp | {level} = DEBUG/INFO/ERROR

# {name} = file ka naam | {message} = actual log message

# <green>, <cyan> = loguru color tags — terminal mein color dikhate hain

format=(

"<green>{time:YYYY-MM-DD HH:mm:ss}</green> | "

"<level>{level: <8}</level> | "

"<cyan>{name}</cyan> | "

"<white>{message}</white>"

),

colorize=True   # Windows terminal mein bhi colors enable karo

)

# ---- Handler 2: File (logs/ folder) ----

# Yeh permanent record ke liye hai — debugging aur audit ke liye

logger.add(

# Har din ki alag file — aeroguard_2025-01-15.log

os.path.join(LOGS_DIR, "aeroguard_{time:YYYY-MM-DD}.log"),

level=LOG_LEVEL,

# File mein color tags nahi — plain text

format=(

"{time:YYYY-MM-DD HH:mm:ss} | "

"{level: <8} | "

"{name} | "

"{message}"

),

rotation="10 MB",       # 10MB hone pe nayi file shuru

retention="30 days",    # 30 din se purani files delete

compression="zip"       # purani files compress karo disk space bachane

)

Step 11 - Now we will test our logger.py with a test code

Ek chhota sa test script chalayenge — verify karenge ki terminal mein colored logs aa rahe hain aur logs/ folder mein file bhi ban rahi hai.

Terminal  python -c "from src.logger import logger; logger.info('AeroGuard logger test'); logger.debug('Debug message test'); logger.error('Error message test')"

Step 12 – Add this “logs” folder and “.env” file to “.gitignore” file (.env might have already been added by default) – we never push our logs to the cloud

Step 13 – Push it to github

git add .

git commit -m "feat1: add logger with console and file logging"

git push origin main

Exceptions

Step 14 – Setting up “Exceptions” for our project

Create a “exception.py” file under the “src” folder and make the basic exception classes as of now

# ============================================================

# AeroGuard — Custom Exception Handler

#

# Yeh file poore project ki custom exception handling karti hai

#

# Problem jo yeh solve karta hai:

# Python ka default error message aisa hota hai:

#   "FileNotFoundError: file.csv not found"

# Isse pata nahi chalta — kaunsi file, kahan, kyun, kab

#

# Hamara custom exception aisa dikhega:

#   "AeroGuardException: [flight_header.csv] not found

#    Location: src/data/ingestion.py, Line 45

#    Context: Loading NGAFID flight header data"

#

# Usage:

#   from src.exception import AeroGuardException

#   raise AeroGuardException("Kuch galat hua", context="Data loading")

# ============================================================

import sys

import traceback

from src.logger import logger

def get_error_details(error: Exception) -> dict:

"""

Kisi bhi exception se poori detail extract karta hai.

Yeh function Python ke sys module se error ki exact location

nikalta hai — kaunsi file, kaunsi line number, kya message.

Args:

error: Jo bhi exception aaya ho

Returns:

dict with file_name, line_number, error_message

"""

# sys.exc_info() teen cheezein deta hai:

# [0] = error type (jaise FileNotFoundError)

# [1] = error object

# [2] = traceback object — yahan exact location hoti hai

exc_type, exc_value, exc_tb = sys.exc_info()

# Agar traceback available hai toh location nikalo

if exc_tb is not None:

# tb_frame = current stack frame

# f_code = code object

# co_filename = file ka path

file_name = exc_tb.tb_frame.f_code.co_filename

line_number = exc_tb.tb_lineno

else:

file_name = "Unknown"

line_number = 0

return {

"file_name": file_name,

"line_number": line_number,

"error_message": str(error),

"error_type": type(error).__name__

}

class AeroGuardException(Exception):

"""

AeroGuard ka master custom exception class.

Poore project mein jahan bhi koi error aaye, yeh class

use hogi taaki consistent aur detailed error messages milein.

Yeh Python ke built-in Exception class ko inherit karta hai —

matlab yeh ek proper Exception hai jo try/except mein

catch ki ja sakti hai.

Example:

try:

load_data("wrong_path.csv")

except Exception as e:

raise AeroGuardException(e, context="Loading flight header")

"""

def __init__(self, error: Exception | str, context: str = ""):

"""

Args:

error   : Original exception ya error message string

context : Kahan kya ho raha tha jab error aaya

Example: "Loading NGAFID flight header data"

"""

# Error details extract karo

if isinstance(error, Exception):

details = get_error_details(error)

else:

# Agar string pass ki toh basic details banao

details = {

"file_name": "Unknown",

"line_number": 0,

"error_message": str(error),

"error_type": "AeroGuardError"

}

# Human-readable error message banao

self.error_message = (

f"\n"

f"{'='*60}\n"

f"  AEROGUARD EXCEPTION\n"

f"{'='*60}\n"

f"  Type    : {details['error_type']}\n"

f"  Message : {details['error_message']}\n"

f"  File    : {details['file_name']}\n"

f"  Line    : {details['line_number']}\n"

f"  Context : {context if context else 'Not provided'}\n"

f"{'='*60}"

)

# Logger se bhi log karo — file aur terminal dono mein jayega

logger.error(self.error_message)

# Parent Exception class ko initialize karo

# Taaki yeh proper Exception ki tarah behave kare

super().__init__(self.error_message)

def __str__(self):

"""

Jab bhi exception print hoga ya str() call hoga

toh yeh formatted message dikhega

"""

return self.error_message

# ============================================================

# Specific Exception Classes

# Har module ke liye alag exception — debugging mein aasani

# ============================================================

class DataIngestionException(AeroGuardException):

"""

Data loading aur ingestion ke dauran aane wali errors.

Example: NGAFID parquet files nahi mili, CSV corrupt hai

"""

pass

class DataValidationException(AeroGuardException):

"""

Data validation ke dauran aane wali errors.

Example: Required columns missing hain, data types galat hain

"""

pass

class DataTransformationException(AeroGuardException):

"""

Feature engineering aur preprocessing ke dauran errors.

Example: Normalization fail hua, NaN handle nahi hua

"""

pass

class ModelTrainingException(AeroGuardException):

"""

Model training ke dauran aane wali errors.

Example: Insufficient data, convergence fail

"""

pass

class ModelPredictionException(AeroGuardException):

"""

Inference/prediction ke dauran aane wali errors.

Example: Model file nahi mili, input shape galat hai

"""

pass

class AnomalyDetectionException(AeroGuardException):

"""

Anomaly detection ke dauran aane wali errors.

Example: Autoencoder reconstruct nahi kar paya

"""

pass

class AlertGenerationException(AeroGuardException):

"""

Alert generate karne ke dauran aane wali errors.

Example: XAI explanation fail hua, severity calculate nahi hua

"""

pass

class ConfigurationException(AeroGuardException):

"""

Config file load karne ya parse karne mein errors.

Example: config.yaml missing hai, required key nahi mili

"""

Pass

Step 14 (b) - Now we will test the “exceptions.py” file using the test code –

First create a “test_exception.py” file in the root folder and paste the code –

from src.exception import AeroGuardException, DataIngestionException

try:

raise ValueError("flight_header.csv not found")

except Exception as e:

raise AeroGuardException(e, context="Loading NGAFID flight header data")

Terminal – python test_exception.py

Step 14(b) – If the code runs successfully then delete this “test_exception.py” file

Step 15 – push to github – Terminal ->

git add .

git commit -m "feat 2: add custom exception handling"

git push origin main

DATA INGESTION -

Step 16 – Create a folder “data” inside the root folder.

Step 17 - Ab data folder ko .gitignore mein add karo — dataset 4.3GB ka hai, yeh kabhi GitHub pe nahi jaana chahiye.

.gitignore kholo aur sabse neeche yeh add karo: /data/

Step 18 – Now create a “data” folder inside the “src” folder.

Step 19 - Ab “src/data” folder ke andar “__init__.py” file banao.

Step 20 – create a file “ingestion.py” inside the “src/data” folder.

ingestion.py ka kaam kya hai:

1. config.yaml se paths padho

2. flight_header.csv load karo pandas mein

3. flight sensor data load karo Dask mein (4.3GB hai isliye Dask)

4. Basic info log karo — kitni flights, kitne columns, etc.

5. Dono dataframes return karo

Code –

# ============================================================

# AeroGuard — Data Ingestion

#

# Kaam: NGAFID dataset ko load karna

#   1. flight_header.csv → pandas DataFrame

#      (metadata — labels, maintenance info, flight duration)

#   2. all_flights/one_parq → Dask DataFrame

#      (actual sensor readings — 4.3GB, 100M+ rows)

#

# Dask kyun?

#   pandas poori file ek saath RAM mein load karta hai

#   4.3GB pandas mein load karna = RAM full = crash

#   Dask lazy loading karta hai — sirf jo zaroorat ho wahi load karega

#

# Usage:

#   from src.data.ingestion import load_data

#   header_df, flight_df = load_data()

# ============================================================

import os

import pandas as pd

import dask.dataframe as dd

import yaml

from src.logger import logger

from src.exception import DataIngestionException

def load_config() -> dict:

"""

config.yaml file load karta hai.

Returns:

dict: saari config settings

Raises:

DataIngestionException: agar config file nahi mili

"""

try:

config_path = "configs/config.yaml"

# Check karo file exist karti hai ki nahi

if not os.path.exists(config_path):

raise FileNotFoundError(f"config.yaml nahi mili: {config_path}")

with open(config_path, "r") as f:

config = yaml.safe_load(f)

logger.debug("config.yaml successfully load hua")

return config

except Exception as e:

raise DataIngestionException(e, context="Loading config.yaml")

def load_flight_header(header_path: str) -> pd.DataFrame:

"""

flight_header.csv load karta hai pandas mein.

Yeh file har flight ka metadata contain karti hai:

- Master Index: flight ka unique ID (sensor data se join key)

- before_maintenance: 1 = maintenance se pehle, 0 = baad mein

- maintenance_type: kaunsa issue (0-35)

- duration: flight kitni der ki thi (seconds)

Args:

header_path: flight_header.csv ka path

Returns:

pd.DataFrame: flight metadata

Raises:

DataIngestionException: agar file nahi mili ya load nahi hua

"""

try:

logger.info(f"Flight header load ho raha hai: {header_path}")

# Check karo file exist karti hai

if not os.path.exists(header_path):

raise FileNotFoundError(

f"flight_header.csv nahi mili: {header_path}\n"

f"Pehle dataset download karo: "

f"https://doi.org/10.5281/zenodo.6624956"

)

# Master Index ko index column set karo

# Yeh sensor data ke saath join karne ke liye use hoga

df = pd.read_csv(header_path, index_col="Master Index")

logger.info(f"Flight header load hua — Shape: {df.shape}")

logger.info(f"Columns: {list(df.columns)}")

logger.info(f"Total flights: {len(df):,}")

return df

except Exception as e:

raise DataIngestionException(e, context="Loading flight_header.csv")

def load_flight_sensor_data(parquet_path: str) -> dd.DataFrame:

"""

Flight sensor data Dask mein load karta hai.

Yeh 4.3GB ka dataset hai — 100M+ rows, 23 sensors.

Pandas mein load karna impossible hai normally.

Dask lazy loading karta hai:

- File abhi RAM mein nahi aati

- Sirf jab .compute() call karo tab load hoti hai

- Ya phir specific flights filter karke load kar sakte hain

Args:

parquet_path: one_parq folder ka path (Dask parquet)

Returns:

dd.DataFrame: Dask DataFrame (lazy loaded)

Raises:

DataIngestionException: agar folder nahi mila

"""

try:

logger.info(f"Flight sensor data load ho raha hai: {parquet_path}")

# Check karo folder exist karta hai

if not os.path.exists(parquet_path):

raise FileNotFoundError(

f"Sensor data folder nahi mila: {parquet_path}\n"

f"Pehle dataset download karo aur unzip karo: "

f"https://doi.org/10.5281/zenodo.6624956"

)

# Dask lazy load — abhi sirf schema padha jaayega

# Actual data tab aayega jab filter ya compute karenge

df = dd.read_parquet(parquet_path)

logger.info(f"Flight sensor data lazy load hua")

logger.info(f"Columns: {list(df.columns)}")

logger.info(f"Partitions: {df.npartitions}")

return df

except Exception as e:

raise DataIngestionException(e, context="Loading flight sensor parquet data")

def load_data() -> tuple[pd.DataFrame, dd.DataFrame]:

"""

Main function — poora dataset load karta hai.

Yeh function config.yaml se paths padh ke

dono files load karta hai aur return karta hai.

Returns:

tuple: (flight_header_df, flight_sensor_df)

- flight_header_df: pandas DataFrame — metadata + labels

- flight_sensor_df: Dask DataFrame — sensor time series

Raises:

DataIngestionException: koi bhi step fail ho toh

"""

try:

logger.info("="*50)

logger.info("AEROGUARD DATA INGESTION SHURU")

logger.info("="*50)

# Step 1: Config load karo

config = load_config()

# Step 2: Paths config se nikalo

header_path = config["data"]["flight_header"]

parquet_path = config["data"]["raw_flight_data"]

logger.info(f"Header path: {header_path}")

logger.info(f"Sensor data path: {parquet_path}")

# Step 3: Flight header load karo

header_df = load_flight_header(header_path)

# Step 4: Sensor data load karo

sensor_df = load_flight_sensor_data(parquet_path)

logger.info("="*50)

logger.info("DATA INGESTION COMPLETE")

logger.info("="*50)

return header_df, sensor_df

except Exception as e:

raise DataIngestionException(e, context="Main data loading pipeline")

Step 21- since the data has 2 folders in itself, therefore paste it under the “data” folder that is in the root directory.

(hum yaha se data fetch karenge aur config file ko bhi issi k according update karenge)

Step 22 – changing the “ingestion.py” file accordingly, since, now we have multiple folders inside the data (root) folder.

Code –

# ============================================================

# AeroGuard — Data Ingestion

#

# Kaam: NGAFID dataset ko load karna

#

# Yeh module project ka pehla real data layer hai.

# Iska kaam hai raw dataset ko disk se read karna aur

# memory-safe format mein ML pipeline ko provide karna.

#

# Teen primary datasets load honge:

#

#   1. all_flights/flight_header.csv

#      → poora dataset metadata (EDA aur fleet-level analysis ke liye)

#

#   2. 2days/flight_header.csv

#      → benchmark subset metadata (paper ka official training subset)

#

#   3. all_flights/one_parq

#      → actual sensor time-series data (100M+ rows)

#

# Extra training files:

#

#   4. 2days/flight_data.pkl

#      → preprocessed numpy arrays (model training ready)

#

#   5. stats.csv (future normalization ke liye)

#

# Dask kyun use kar rahe hain?

#

# Sensor dataset ~4.3GB hai.

# Agar pandas se load karenge toh poori file RAM mein aayegi.

# Dask lazy evaluation karta hai:

#

#   - sirf metadata read hota hai

#   - data tab load hota hai jab compute() call hota hai

#

# Isse large dataset safely process kar sakte hain.

#

# Usage Example:

#

#   from src.data.ingestion import load_data

#

#   data = load_data()

#

#   data["header_full"]        → poora flight metadata

#   data["header_2days"]       → benchmark subset

#   data["sensor_data"]        → Dask DataFrame

#   data["flight_data_2days"]  → numpy arrays

#

# ============================================================

import os

import pickle

import pandas as pd

import dask.dataframe as dd

import yaml

from src.logger import logger

from src.exception import DataIngestionException

# ------------------------------------------------------------

# CONFIG LOADER

# ------------------------------------------------------------

# Yeh function config.yaml ko load karta hai.

# Config file project ke saare paths aur parameters define karti hai.

# Hardcoding avoid karne ke liye har module config se values padhta hai.

# ------------------------------------------------------------

def load_config() -> dict:

try:

config_path = "configs/config.yaml"

# Check karo config file exist karti hai ya nahi

# Agar nahi mili toh pipeline immediately fail karegi

if not os.path.exists(config_path):

raise FileNotFoundError(

f"config.yaml nahi mili: {config_path}"

)

# YAML ko Python dictionary mein convert karo

with open(config_path, "r") as f:

config = yaml.safe_load(f)

logger.debug("config.yaml successfully load hua")

return config

except Exception as e:

raise DataIngestionException(e, context="Loading config.yaml")

# ------------------------------------------------------------

# FULL FLIGHT HEADER LOADER

# ------------------------------------------------------------

# Yeh function poora metadata dataset load karta hai.

#

# Use cases:

#   - Exploratory Data Analysis (EDA)

#   - Cross-flight trend analysis

#   - Fleet level statistics

#

# Dataset size ~28k flights

# ------------------------------------------------------------

def load_flight_header_full(header_path: str) -> pd.DataFrame:

try:

logger.info(f"Full flight header load ho raha hai: {header_path}")

# File existence check

if not os.path.exists(header_path):

raise FileNotFoundError(

f"Full flight header nahi mila: {header_path}"

)

# Master Index ko DataFrame index bana rahe hain

# Yeh unique flight identifier hai

# Sensor dataset ke cluster column se match hoga

df = pd.read_csv(header_path, index_col="Master Index")

logger.info(f"Full flight header load hua — Shape: {df.shape}")

logger.info(f"Columns: {list(df.columns)}")

logger.info(f"Total flights: {len(df):,}")

# Maintenance distribution check

if "before_after" in df.columns:

# Maintenance se pehle wali flights

logger.info(

f"Before maintenance: {(df['before_after']==1).sum():,} flights"

)

# Maintenance ke baad wali flights

logger.info(

f"After maintenance: {(df['before_after']==0).sum():,} flights"

)

return df

except Exception as e:

raise DataIngestionException(

e, context="Loading full flight_header.csv"

)

# ------------------------------------------------------------

# BENCHMARK HEADER LOADER

# ------------------------------------------------------------

# Yeh subset dataset hai jo research paper mein use hua tha.

#

# Sirf woh flights include hoti hain jo maintenance event

# ke 2 din ke window mein aati hain.

#

# Yeh dataset model training ke liye ideal hai kyunki

# labels already curated hain.

# ------------------------------------------------------------

def load_flight_header_2days(header_path: str) -> pd.DataFrame:

try:

logger.info(f"2days flight header load ho raha hai: {header_path}")

if not os.path.exists(header_path):

raise FileNotFoundError(

f"2days flight header nahi mila: {header_path}"

)

df = pd.read_csv(header_path, index_col="Master Index")

logger.info(f"2days flight header load hua — Shape: {df.shape}")

logger.info(f"Columns: {list(df.columns)}")

logger.info(f"Total flights: {len(df):,}")

return df

except Exception as e:

raise DataIngestionException(

e, context="Loading 2days flight_header.csv"

)

# ------------------------------------------------------------

# SENSOR DATA LOADER (DASK)

# ------------------------------------------------------------

# Yeh sabse bada dataset hai — 100M+ rows.

#

# Isme har flight ka per-second sensor data hota hai.

#

# Typical columns:

#   - Engine sensors

#   - Fuel system sensors

#   - Electrical system sensors

#   - Flight dynamics sensors

#

# Dask lazy load karta hai:

#

#   df = dd.read_parquet(...)

#

# Is step mein sirf schema read hota hai.

# Actual data tab load hota hai jab compute() call hota hai.

# ------------------------------------------------------------

def load_flight_sensor_data(parquet_path: str) -> dd.DataFrame:

try:

logger.info(

f"Flight sensor data lazy load ho raha hai: {parquet_path}"

)

if not os.path.exists(parquet_path):

raise FileNotFoundError(

f"Sensor data folder nahi mila: {parquet_path}"

)

# Lazy load — memory safe

df = dd.read_parquet(parquet_path)

logger.info(f"Sensor data lazy load hua")

logger.info(f"Columns: {list(df.columns)}")

logger.info(f"Total partitions: {df.npartitions}")

return df

except Exception as e:

raise DataIngestionException(

e, context="Loading flight sensor parquet data"

)

# ------------------------------------------------------------

# PREPROCESSED TRAINING DATA LOADER

# ------------------------------------------------------------

# Yeh paper ka ready-made processed dataset hai.

#

# flight_data.pkl mein numpy arrays store hote hain:

#

# Example:

#   X_train

#   y_train

#   X_val

#

# Pickle format use kiya gaya hai kyunki numpy arrays

# efficiently store ho jaate hain.

# ------------------------------------------------------------

def load_2days_flight_data(pkl_path: str) -> dict:

try:

logger.info(f"2days flight data pkl load ho raha hai: {pkl_path}")

if not os.path.exists(pkl_path):

raise FileNotFoundError(

f"flight_data.pkl nahi mila: {pkl_path}"

)

# Pickle binary format read kar rahe hain

with open(pkl_path, "rb") as f:

data = pickle.load(f)

logger.info(f"flight_data.pkl load hua — Type: {type(data)}")

if isinstance(data, dict):

logger.info(f"Total flights in pkl: {len(data):,}")

logger.debug(f"Sample flight IDs: {list(data.keys())[:10]}")

return data

except Exception as e:

raise DataIngestionException(

e, context="Loading 2days flight_data.pkl"

)

# ------------------------------------------------------------

# MAIN DATA INGESTION PIPELINE

# ------------------------------------------------------------

# Yeh AeroGuard ka main entrypoint hai data loading ke liye.

#

# Pipeline flow:

#

#   config load

#       ↓

#   full metadata load

#       ↓

#   benchmark metadata load

#       ↓

#   sensor parquet lazy load

#       ↓

#   preprocessed training data load

#

# Final output dictionary return karta hai.

# ------------------------------------------------------------

def load_data() -> dict:

try:

logger.info("="*55)

logger.info("AEROGUARD DATA INGESTION SHURU")

logger.info("="*55)

# Step 1 — config load

config = load_config()

# Step 2 — full metadata

header_full = load_flight_header_full(

config["data"]["flight_header_full"]

)

# Step 3 — benchmark subset metadata

header_2days = load_flight_header_2days(

config["data"]["flight_header_2days"]

)

# Step 4 — sensor dataset (lazy)

sensor_data = load_flight_sensor_data(

config["data"]["raw_flight_data"]

)

# Step 5 — preprocessed training arrays

flight_data_2days = load_2days_flight_data(

config["data"]["flight_data_2days"]

)

logger.info("="*55)

logger.info("DATA INGESTION COMPLETE ✓")

logger.info("="*55)

return {

"header_full": header_full,

"header_2days": header_2days,

"sensor_data": sensor_data,

"flight_data_2days": flight_data_2days,

"config": config

}

except Exception as e:

raise DataIngestionException(

e, context="Main data loading pipeline"

)

Step 23 - Test code for “ingestion.py” – Terminal ->

python -c "from src.data.ingestion import load_data; data = load_data(); print('Keys:', list(data.keys()))"

Step 24 – push it to github

git add .

git commit -m "feat 3: data ingestion complete — all 4 files loading successfully"

git push origin main

EXPLORATORY DATA ANALYSIS

Pehle jupyter notebook mai karenge and then ussi analysis ko modular code mai convert kar lenge

Step 25 – Install the ipykernel using – “pip install jupyter ipykernel”

Step 26 - python -m ipykernel install --user --name=aeroguard --display-name "AeroGuard (venv)"

Yeh Command Kya Karta Hai?

Jab tum Jupyter notebook khologe — usmein ek "kernel" select karna hota hai. Kernel matlab — kaunsa Python use karna hai notebook ke liye.

Problem yeh hai ki agar yeh command nahi chalate toh Jupyter apna system Python use karega — jisme hamare project ke packages nahi hain (loguru, dask, pandas etc.)

Yeh command hamara venv wala Python Jupyter mein register karta hai — taaki notebook mein bhi wahi environment use ho jisme humne sab install kiya hai.

Practically speaking –

Bina is command ke:

Notebook → System Python → import dask → ERROR (not found)

Is command ke baad:

Notebook → AeroGuard (venv) → import dask → ✅ works

Step 27 – create a folder named as “notebooks” under the root directory and inside this “notebooks” folder create a new file named as “01_EDA.ipynb”

Step 28 – choose the “kernel” > python environments > aviation wala jo bhi ho

Step 29 – Do the full EDA in your jupyter notebook